KLE TECHNOLOGICAL UNIVERSITY  ×  NVIDIA  |  SENIOR DESIGN PROJECT 2026–27

Develop and Validate Optimised Multimodal Models for Automotive Use Cases

7-Minute Slide-by-Slide Spoken Presentation Script (Bullet-Wise Delivery)

| NVIDIA Guides: Gaurav Tendolkar, Pradipt Trivedi | Project Guide: Dr. Malathi S Y |
| --- | --- |
| Team Members: Saanvi Shetty, Raveesh Nayak, Ramya Rao, Shreyas Kulkarni | Total Timing: 7 Minutes (~1 min / slide + Q&A buffer) |

| Slide 1: Title & Team Introduction | Allocated Time: ~1:00 min |
| --- | --- |

Greeting & Title: "Good morning / afternoon respected guides and panel members. We are presenting our project: Develop and Validate Optimised Multimodal Models for Automotive Use Cases."

Mentorship & Affiliation: "This project is carried out under the valuable mentorship of our NVIDIA guides, Gaurav Tendolkar and Pradipt Trivedi, and our college project guide, Dr. Malathi S Y at KLE Technological University."

Team Introduction: "Our project team consists of Saanvi Shetty, Raveesh Nayak, Ramya Rao, and myself, Shreyas Kulkarni."

The High-Level Motivation: "Modern vehicles need continuous surround-view monitoring while parked, but running heavy Vision Transformers exhausts battery power. Our goal is to eliminate temporal frame redundancy to achieve low-power, high-accuracy surveillance perception on embedded automotive hardware."

| Slide 2: Problem Statement & Key Quality KPIs | Allocated Time: ~1:00 min |
| --- | --- |

Automotive Context: "Our focus application is Automotive Surveillance Mode (ASM), monitoring the vehicle perimeter via four surround fisheye cameras."

The Computational Bottleneck: "Vision Transformers like DINOv2 were trained on static images. Processing continuous 30 FPS video across 4 cameras causes battery drain and exceeds the thermal capacity of the embedded processor."

The Motion & Token Challenge: "Between consecutive frames, moving pedestrians shift spatial positions. Static same-position token matching fails, which requires a motion-aware spatiotemporal tracking solution."

Quality KPIs & Benchmarks: "We benchmark on the open-source VIRAT surveillance dataset, establishing localized action recognition via spatiotemporal tubelet pooling and actor trajectory tracking using Adapter-v2 + YOLO26."

| Slide 3: High-Level Optimised Multimodal Architecture | Allocated Time: ~1:15 min |
| --- | --- |

Camera Feeds & Temporal Decimation: "Starting on the left: four 30 FPS fisheye feeds enter the pipeline. We first apply native Temporal Decimation to 5 FPS, cutting 83.3% of redundant frames immediately before model inference."

Shared DINOv2 Backbone (ViT-S/14): "A single frozen 22M-parameter backbone processes the 5 FPS stream, outputting spatial patch tokens and multi-layer hidden states for all downstream tasks with zero redundant passes."

Head A — Actor Detection & Tracking: "Adapter-v2 with YOLO detects pedestrians and vehicles, and a multi-frame Hungarian tracker builds continuous bounding-box actor trajectories across time."

Head B — VIRAT Action Head: "We pool spatiotemporal tubelet features from the DINOv2 patch tokens and pass them to a 2-layer temporal self-attention Transformer to classify actions like walking, standing, or approaching vehicles."

Embedded PVA Hardware Target: "Both streams converge into the Low-Power PVA processor and Threat Assessment engine, fulfilling our strict compute budget of < 10 GMACs per frame to preserve battery State of Charge."

| Slide 4: Multimodal Optimization & Cross-Scene Validation Strategy | Allocated Time: ~1:00 min |
| --- | --- |

Why Leave-One-Scene-Out (LOSO) Validation: "To verify that our model generalizes to real-world automotive environments, we instituted a strict 3-Fold Leave-One-Scene-Out cross-validation protocol."

Cross-Scene Split Design: "Each fold trains on two scenes and evaluates exclusively on a completely unseen third surveillance location with distinct camera angles, elevations, and lighting conditions."

Embedding Noise Injection & Augmentation: "We injected Gaussian feature noise (sigma = 0.04) and applied 15% feature dropout to prevent over-reliance on static background features."

Regularization & Loss Calibration: "We implemented 0.10 label smoothing and validation early stopping to guarantee calibrated, reliable prediction probabilities for vehicle threat alerts."

| Slide 5: Performance Benchmarking: Baseline vs. Optimized Multimodal Model | Allocated Time: ~1:15 min |
| --- | --- |

Top-Line Metric Highlights: "A standard global Kinetics-400 linear probe achieves only 22.4% accuracy due to a complete lack of spatial actor awareness."

Compute Decimation Impact: "Our native 5 FPS decimation provides an 83.3% forward-compute saving across the surround-view camera feeds."

Unseen Cross-Scene Generalization: "On genuinely unseen surveillance scenes, our optimized multimodal model achieves 70.83% Top-1 Accuracy and a 69.16% Weighted F1-Score."

Surveillance Action Discrimination: "On primary surveillance actions, our spatiotemporal tubelet pooling achieves 75.0% F1-score on Walking (81.8% recall) and 69.6% F1-score on Standing (72.7% precision)."

Key Takeaway: "Coupling spatial actor bounding-box trajectories with temporal self-attention delivers strong, generalizable accuracy under diverse surveillance conditions."

| Slide 6: Project Milestones & Implementation Roadmap | Allocated Time: ~1:00 min |
| --- | --- |

Milestone 1 — Achieved: "Integrated the open-source VIRAT surveillance benchmark, deployed the 5 FPS decimation pipeline, and developed the Tubelet Transformer action head."

Milestone 2 — Achieved: "Completed 3-Fold LOSO cross-scene validation, confirmed 70.83% unseen Top-1 accuracy, and exported our calibrated production checkpoint (best.pth)."

Upcoming Phase — Low-Power PVA Target: "We will implement motion-aware token pruning and frame differencing to dynamically skip static background tokens across frames, ensuring the active compute stays strictly below our < 10 GMACs per frame KPI."

Closing for Q&A: "Thank you. We are now happy to answer any questions from our guides and the panel."

Presentation Timing & Delivery Tips: • Pacing: Keep a steady cadence of roughly 130-140 words per minute; each bullet corresponds to a single clear breath. • Transitions: Pause 1-2 seconds when switching slides to let the panel visually digest the clean graphics. • Buffer: The script covers ~6 minutes of active speaking, leaving a comfortable 1-minute buffer for introduction or questions.